Unit Conversion and Some Important Constants – thermofluids.net

(A more comprehensive resource is the Unit Converter daemon)

Length/Velocity:

1 Foot = 0.3048 m;		1 inch = 25.4 mm		1 mile = 1.61 km

1 mile = 5280 Feet		1 mile (nautical) = 1.85 km

1 mile/hour = 0.447 m/s	1 km/hour = 0.2777 m/s

Volume/Flow Rate:

1 Cubic Feet = 0.02832 m3	1 Gallon = 3.785 l		1 l = 1x10-3 m3

1 Gallon = 0.1336 ft3		1 in3 = 1.6387x10-5 m3		1 Quart = 0.9464 l

1 Quart = 0.25 Gallon		1 Ounce = 29.574 ml		1 Pint = 0.473 l

1 Gallon/hour = 3.711x10-5 ft3/s	1 G/hr=1.0514x10-6 m3/s

Mass:

1 Slug = 14.594 kg	1 lbm = 0.4536 kg	1 Ton (long) = 1016 kg

1 Ounce = 28.35 g

Force:

1 Pound-Force = 4.448 N 	1 kg-Force = 9.81 N	1 Ounce-Force = 0.278 N

1 Dyne = 10 mN			1 KIP = 4448.2 N

Pressure:

1 psi = 6.895 kPa 		1 bar = 100 kPa		1 inch of Hg = 3.374 kPa

1 inch of water = 0.2486 kPa	1 mm of Hg = 0.1333 kPa	1 Torr = 1 mm of Hg

Energy: (1 kW-Hr cost US$0.10 and 1 Therm (CH4) US$0.50 cents in the year 1995)

1 BTU = 1.055 kJ			1 Erg = 1 mJ		1 Therm = 105.5 MJ (59¢)

1 Cal(food) = 4.187 kJ 		1 calorie = 4.187 J	1 kW-Hr = 3.6 MJ (10¢)

1 kg(kilo)Calorie = 4.187 kJ  (in USA Calorie means a Food Calorie = 4.187 kJ)

1 Horsepower-Hour = 2.6845 MJ	1 Ft-Pound force = 1.3558 J

1 BTU/ft3 = 0.0373 MJ/m3	1 BTU/lbm = 2.3258 kJ/kg	1 cal/g = 4.187 kJ/kg

1 BTU/Hour = 0.2931 W		1 Horsepower = 0.7457 kW

1 BTU=778 ft-lbf/lbmol-R	1 Ton of Refrigeration = 3.517 kW

1 MJ of Nat. Gas = 0.56¢	1 MJ of Electricity = 2.78¢ (5 times pricier than gas)

Temperature:

oC = 5*(oF-32)/9		oR = oF+460		K = oC+273

Viscosity:

1 Centipoise = 1 mPa.s (absolute)	1 Centistoke = 1 mm2/s (kinematic)

---------------------------CONSTANTS (within reasonable accuracy)----------------

Quantity				SI			English

Standard temperature		25oC (298K),		60oF (520R)

Standard pressure of air	101 kPa		14.7 psia

g					9.81 m/s2,		32.2 ft/s2

R_bar					8.314 kJ/kg.Kmol	1545.3 ft-lbf/lbmol-R

Mol wt (kg/kmol): 		Air=29; CO2=44; N2=28; O2=32;

C_p,air				1.005 kJ/kg.K,	0.24 BTU/lbm-R

kair = CP/Cv			1.4

Density of water			103 kg/m3		62.4 lbm/ft3

Density of air*			1.2 kg/m3		0.075 lbm/ft3

SG of Hg				13.6 			13.6

Kinematic viscosity of water	1.22x10-5 ft2/s	1.13x10-6 m2/s

Enthalpy of water at 100 oC	419-2676 kJ/kg	180-1150 Btu/lb