Unit Conversion (English to SI) and Some Important Constants

(A more comprehensive resource is the Converter TESTcalc or the Android app Engineering Unit Converter)

Length/Velocity:

1 ft = 0.3048 m;	1 in = 25.4 mm	1 mm = 1x10-3 m

1 mile = 1.61 km	1 mile (nautical) = 1.85 km

1 mile/hour = 0.447 m/s	1 km/hour = 0.2777 m/s

Volume/Flow Rate:

1 ft3 = 0.02832 m3	1 gal = 3.785 L	1 L = 1x10-3 m3

1 in3 = 16.387 mL	1 mL = 1x10-3 L 	1 quart = 0.9464 L

1 ounce = 29.574 mL	1 pint = 0.473 L

1 ft3/min(cfm) = 4.72x10-4 m3/s	1 gal/hour=1.0514x10-6 m3/s

Mass:

1 slug = 14.594 kg	1 lbm = 0.4536 kg	1 g = 1x10-3 kg

1 ounce = 28.35 g

1 Ton (long) = 1016 kg	1 Ton (short) = 2000 lbm = 907.1847 kg

Force:

1 lbf = 4.448 N 	1 kgf = 9.81 N	1 N = 1x10-3 kN

1 Ounce-Force = 0.278 N	1 Dyne = 10 mN	1 KIP = 4.44.2 kN

Pressure:

1 psi = 6.895 kPa 	1 bar = 100 kPa	1 inch of Hg = 3.374 kPa

1 inch of water = 0.2486 kPa	1 Torr = 1 mm of Hg	1 mm of Hg = 0.1333 kPa

Energy/Power:

1 Btu = 1.055 kJ	1 MJ = 1x103 kJ	1 J = 1x10-3 kJ

1 erg = 1 mJ	1 Therm = 105.5 MJ	1 kWh = 3.6 MJ

1 Cal (food) = 4.187 kJ 	1 calorie = 4.187 J	1 kilo-calorie = 4.187 kJ

1 Horsepower-hour = 2.6845 MJ	1 ft-lbf = 1.3558 J

1 Btu/ft3 = 0.0373 MJ/m3	1 Btu/lbm = 2.3258 kJ/kg	1 cal/g = 4.187 kJ/kg

1 Btu/hour = 0.2931 W	1 Horsepower = 0.7457 kW

1 Ton of Refrigeration = 3.517 kW

Temperature:

Specific Heat:

1  =1  = 1

1  =4.187

1  = 4.187

1  = 4.187

Thermal Conductivity:

1  =1  = 1x10-3

1  = 1.7307

Absolute and Kinematic Viscosity:

1 Pa.s = 1 N.s/m2 = 1 	(absolute)

1 poise = 0.1 Pa.s 	1  = 1.4882 Pa.s (absolute)

1 stoke = 100 m2/s (kinematic)

---------------------------Physical Constants----------------

Quantity	SI Unit

Standard temperature	25oC (298K),

Standard pressure of air	101.325 kPa

9.81 m/s2,

8.314

Boltzmann constant	1.380650x10-26 kJ/K

Avogadro constant	6.0221413x1026 per kmol

Stephan-Boltzmann constant	5.6704x10-11

Speed of light in vacuum	2.9979x108 m/s

Speed of sound in dry air (1 atm, 0oC)	331.36 m/s

Density of air (STP)	1.185 kg/m3

Density of water (STP)	997 kg/m3

Kinematic viscosity of water (20oC)	1.002 Pa s

Heat of fusion of water (1 atm)	333.7 kJ/kg

Enthalpy of vaporization of water (1 atm)	2256.5 kJ/kg